Big picture Logisim circuit

Main
ALU
Register_File
Control_Unit
Data_Memory
Instruction_Memory
Flag_Register
PC_Logic

PC 
↓
Instruction Memory
↓
Instruction Splitter
↓
Control Unit ──────────┐
↓                                                │
Register File                         │
↓                                                │
ALU / Data Memory          │
↓                                                │
Write Back to Register File 
↓
Flags update

PC Logic decides:
PC = PC + 1
or
PC = Jump Address
or
PC = Branch Address
or
Stop if HLT

Program Counter

The PC is an 8-bit register.

It stores the address of the current instruction.

Example:

| PC value | Instruction being executed |
| --- | --- |
| 00 | First instruction |
| 01 | Second instruction |
| 02 | Third instruction |

Because each instruction is already 16 bits wide, the PC increases by 1, not by 2.

So normally:

PC next = PC + 1

For jump:

PC next = address

For halt:

PC does not change

. Decoding the Opcode (The Secret Language)

Looking closely at the second image, the "Opcode" column is a grid of black and white squares. In digital logic, Black = 1 and White = 0. These 4 bits tell the processor what math or logic operation to perform. I have decoded them for you:

- 0000: NOP (No Operation - do nothing)

- 0001: HLT (Halt - stop the clock/program)

- 0010: ADD (Add A + B)

- 0011: SUB (Subtract A - B)

- 0100: NOR (Bitwise NOR)

- 0101: AND (Bitwise AND)

- 0110: INC (Increment A + 1)

- 0111: DEC (Decrement A - 1)

- 1000: RSH (Right Shift A >> 1)

- 1001: LDI (Load Immediate - put a specific number directly into a register)

- 1010: LOD (Load from memory into a register

- 1011: STR (Store register value into memory)

- 1100: JMP (Jump to a new line of code)

- 1101: BIF (Branch/Jump if a flag condition is met)

 Registers (The Workbench): Fast, temporary storage inside the CPU. You have 8 of these (R0 to R7). Think of them like temporary variables.

- Special Rule: R0 is always exactly 0.

- Special Rule: R7 holds the address when you want to talk to Data Memory.

  ALU (Arithmetic Logic Unit - The Calculator): This is the circuit that actually does the ADD, SUB, AND, NOR, etc.

  Memory (The Library): You have a Harvard Architecture, which is just a fancy way of saying your code (Instruction Memory) is kept in a separate box from your variables/data (Data Memory).

  Program Counter (The Bookmark): A counter that simply holds the line number of the code you are currently executing.

  Flags (The Status Lights): 3 little bulbs that light up based on the ALU's last calculation: Zero (Result was 0), Carry (Result was too big for 8 bits), and Negative (Result is below zero).